KV260 AUTONOMOUS VEHICLE VISION SYSTEM

| CARLA SIMULATOR Windows PC / Python • Car / pedestrian / traffic • RGB Camera 640×480 • ~10 FPS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | JPEG frames TCP Ethernet Port 5000 |  |  |  |  |  |
| KV260 PetaLinux ARM Cortex-A53 CPU kv260-vision-network-app OpenCV does: • JPEG decoding • image preprocessing • drawing results • lane post-processing • video recording |  |  |  |  |  |  |
|  |  |  | Frame goes to FPGA hardware |  |  |  |  |  |
| KV260 PROGRAMMABLE LOGIC FPGA Vitis AI DPU YOLOv5 Nano Object Detection • car • person • bicycle • bus / truck etc.. Custom HLS Sobel IP Edge Detection. AXI + DDR Memory Connections |
|  |  |  | Results return to ARM |  |  |  |  |  |
| ARM / OpenCV FUSION YOLO result + Sobel edges ↓ OpenCV lane candidate process ↓ Final combined visualization |  |  |  |  |  |  |
| LIVE BROWSER OUTPUT 192.168.50.2:8080 |  |  |  |  |  | VIDEO RECORDING kv260_carla_live.avi |  |  |  |